EXP 6

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

use IEEE.STD_LOGIC_arith.ALL;

use IEEE.STD_LOGIC_unsigned.ALL;

entity usr1 is

Port ( Clk, rst, sin : in STD_LOGIC;

m : in STD_LOGIC_VECTOR (1 downto 0);

pin : in STD_LOGIC_VECTOR (3 downto 0);

sout : out STD_LOGIC;

pout : out STD_LOGIC_VECTOR (3 downto 0));

end usr1;

architecture Behavioral of usr1 is

signal p1,p2:std_logic_vector(3 downto 0);

signal s1:std_logic;

begin

process(clk,rst)

begin

if(rst='1') then

p1 <= "0000";

p2 <= "0000";

s1 <= '0';

elsif (clk'event and clk='1') then

if(m="00") then --SISO

p1(0) <= sin;

p1(1) <= p1(0);

p1(2) <= p1(1);

p1(3) <= p1(2);

s1 <= p1(3);

elsif(m="01") then --SIPO

p1(0) <= sin;

p1(1) <= p1(0);

p1(2) <= p1(1);

p1(3) <= p1(2);

p2 <= p1;

elsif(m="10") then --PIPO

p1 <= pin;

p2 <= pin;

else --PISO

p1(3) <= p1(2);

p1(2) <= p1(1);

p1(1) <= p1(0);

p1(0) <= '0';

s1 <= p1(3);

end if;

end if;

end process;

pout <= p2;

sout <= s1;

end Behavioral;

Test bench code

library ieee;

use ieee.std_logic_1164.ALL;

entity usr_tb is

end usr_tb;

architecture behavior of usr_tb is

-- Component Declaration

component usr1

port(

Clk : in std_logic;

rst : in std_logic;

sin : in std_logic;

m : in std_logic_vector(1 downto 0);

pin : in std_logic_vector(3 downto 0);

sout : out std_logic;

pout : out std_logic_vector(3 downto 0)

);

end component;

-- Inputs

signal Clk : std_logic := '0';

signal rst : std_logic := '0';

signal sin : std_logic := '0';

signal m : std_logic_vector(1 downto 0) := (others => '0');

signal pin : std_logic_vector(3 downto 0) := (others => '0');

-- Outputs

signal sout : std_logic;

signal pout : std_logic_vector(3 downto 0);

-- Clock period

constant Clk_period : time := 20 ns;

begin

-- UUT Instantiation

uut: usr1 port map (

Clk => Clk,

rst => rst,

sin => sin,

m => m,

pin => pin,

sout => sout,

pout => pout

);

-- Clock process

Clk_process : process

begin

Clk <= '0';

wait for Clk_period/2;

Clk <= '1';

wait for Clk_period/2;

end process;

-- Reset process

rst_process : process

begin

rst <= '1';

wait for Clk_period;

rst <= '0';

wait;

end process;

-- Stimulus process

stim_proc: process

begin

-- SISO

pin <= "1001";

sin <= '1';

m <= "00";

wait for 120 ns;

-- SIPO

pin <= "1101";

sin <= '1';

m <= "01";

wait for 120 ns;

-- PIPO

pin <= "1101";

m <= "10";

wait for 120 ns;

-- PISO

pin <= "0100";

m <= "11";

wait for 120 ns;

wait;

end process;

end;