EXP 7 FIFO using VHDL

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

use IEEE.STD_arith_.ALL;

use IEEE.STD_unsigned_.ALL;

entity fifo is

generic (depth : integer := 8);

port (

clk : in std_logic;

reset : in std_logic;

enr : in std_logic;

enw : in std_logic;

data_in : in std_logic_vector (2 downto 0);

data_out : out std_logic_vector(2 downto 0);

fifo_empty : out std_logic;

fifo_full : out std_logic

);

end fifo;

architecture Behavioral of fifo is

type memory_type is array (0 to depth-1) of std_logic_vector(2 downto 0);

signal memory : memory_type := (others => (others => '0'));

signal readptr, writeptr : integer range 0 to depth-1 := 0;

signal empty, full : std_logic := '1';

signal dataout : std_logic_vector(2 downto 0);

signal count : integer range 0 to depth := 0;

begin

fifo_empty <= empty;

fifo_full <= full;

data_out <= dataout;

process(clk, reset)

begin

if(reset = '1') then

dataout <= (others => '0');

readptr <= 0;

writeptr <= 0;

count <= 0;

empty <= '1';

full <= '0';

elsif rising_edge(clk) then

-- WRITE

if(enw = '1' and full = '0') then

memory(writeptr) <= data_in;

if(writeptr = depth-1) then

writeptr <= 0;

else

writeptr <= writeptr + 1;

end if;

count <= count + 1;

end if;

-- READ

if(enr = '1' and empty = '0') then

dataout <= memory(readptr);

if(readptr = depth-1) then

readptr <= 0;

else

readptr <= readptr + 1;

end if;

count <= count - 1;

end if;

-- FLAGS

if(count = 0) then

empty <= '1';

else

empty <= '0';

end if;

if(count = depth) then

full <= '1';

else

full <= '0';

end if;

end if;

end process;

end Behavioral;

Test Bench Code
library ieee;

use ieee.std_logic_1164.ALL;

use ieee.numeric_std.ALL;

entity fifo_tb is

end fifo_tb;

architecture behavior of fifo_tb is

-- Inputs and outputs

signal Clk, reset, enr, enw : std_logic := '0';

signal fifo_empty, fifo_full : std_logic;

signal data_in, data_out : std_logic_vector(2 downto 0) := (others => '0');

-- Clock period definitions

constant Clk_period : time := 10 ns;

constant depth : integer := 8;

begin

-- Instantiate the Unit Under Test (UUT)

uut: entity work.fifo

generic map (depth => depth)

port map (

clk => Clk,

reset => reset,

enr => enr,

enw => enw,

data_in => data_in,

data_out => data_out,

fifo_empty => fifo_empty,

fifo_full => fifo_full

);

-- Clock process

Clk_process : process

begin

Clk <= '0';

wait for Clk_period/2;

Clk <= '1';

wait for Clk_period/2;

end process;

-- Stimulus process

stim_proc: process

begin

-- Apply reset

reset <= '1';

wait for Clk_period;

reset <= '0';

wait for Clk_period*2;

-- WRITE (fill FIFO)

enw <= '1';

enr <= '0';

for i in 1 to depth loop

data_in <= std_logic_vector(to_unsigned(i,3));

wait for Clk_period;

end loop;

-- STOP WRITE

enw <= '0';

wait for Clk_period;

-- READ (empty FIFO)

enr <= '1';

for i in 1 to depth loop

wait for Clk_period;

end loop;

-- STOP READ

enr <= '0';

wait;

end process;

end behavior;

Implementation Code
library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

architecture Behavioral of fifo is

type memory_type is array (0 to depth-1) of std_logic_vector(2 downto 0);

signal memory : memory_type :=(others => (others => '0'));

signal readptr,writeptr : integer := 0;

signal empty,full : std_logic := '0';

signal temp:std_logic_vector(27 downto 0);

signal sclk:std_logic;

signal dataout:std_logic_vector(2 downto 0);  -- ✅ fixed

begin

p11: process(clk,reset)

begin

if(reset='1') then

temp <= (others=>'0');

elsif rising_edge(clk) then

temp <= std_logic_vector(unsigned(temp) + 1);  -- ✅ fixed

end if;

end process;

sclk <= temp(27);

end Behavioral;

Constraint File Code. Ucf

NET "clk" LOC = "L15" | IOSTANDARD = LVCMOS33;

NET "reset" LOC = "E4" | IOSTANDARD = LVCMOS33;

NET "enr" LOC = "T5" | IOSTANDARD = LVCMOS33;

NET "enw" LOC = "R5" | IOSTANDARD = LVCMOS33;

NET "data_in<0>" LOC = "A10" | IOSTANDARD = LVCMOS33;

NET "data_in<1>" LOC = "D14" | IOSTANDARD = LVCMOS33;

NET "data_in<2>" LOC = "C14" | IOSTANDARD = LVCMOS33;

NET "data_out<0>" LOC = "U18" | IOSTANDARD = LVCMOS33;

NET "data_out<1>" LOC = "M14" | IOSTANDARD = LVCMOS33;

NET "data_out<2>" LOC = "N14" | IOSTANDARD = LVCMOS33;

NET "fifo_empty" LOC = "P16" | IOSTANDARD = LVCMOS33;

NET "fifo_full" LOC = "D4" | IOSTANDARD = LVCMOS33;