EXP 8
Model and verify Modulo-N-Counter (MOD_N)

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity modN_counter is

generic(n: integer := 128);

port(

clk : in STD_LOGIC;

reset : in STD_LOGIC;

dout : out STD_LOGIC_VECTOR(6 downto 0)

);

end modN_counter;

architecture A_modN of modN_counter is

begin

counter : process(clk, reset)

variable m : integer range 0 to n-1 := 0;

begin

if (reset = '1') then

m := 0;

elsif rising_edge(clk) then

if (m = n-1) then

m := 0;

else

m := m + 1;

end if;

end if;

dout <= std_logic_vector(to_unsigned(m,7));

end process;

end A_modN;



Test bench code

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

USE ieee.numeric_std.ALL;

ENTITY MOD_TB IS

END MOD_TB;

ARCHITECTURE behavior OF MOD_TB IS

-- Component Declaration

COMPONENT modN_counter

PORT(

clk : IN std_logic;

reset : IN std_logic;

dout : OUT std_logic_vector(6 downto 0)

);

END COMPONENT;

-- Inputs

signal clk : std_logic := '0';

signal reset : std_logic := '0';

-- Outputs

signal dout : std_logic_vector(6 downto 0);

-- Clock period

constant clk_period : time := 10 ns;

BEGIN

-- UUT

uut: modN_counter PORT MAP (

clk => clk,

reset => reset,

dout => dout

);

-- Clock process

clk_process : process

begin

clk <= '0';

wait for clk_period/2;

clk <= '1';

wait for clk_period/2;

end process;

-- Stimulus process

stim_proc: process

begin

-- Reset

reset <= '1';

wait for 20 ns;

reset <= '0';

-- Let counter run

wait for 200 ns;

wait;

end process;

END;

Impleentation Code:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity modN_counter is

generic(n: integer := 128);

port(

clk : in STD_LOGIC;

reset : in STD_LOGIC;

dout : out STD_LOGIC_VECTOR(6 downto 0)

);

end modN_counter;

architecture A_modN of modN_counter is

signal temp : std_logic_vector(22 downto 0);

signal sclk : std_logic;

begin

-- Clock divider

p11: process(clk, reset)

begin

if(reset = '1') then

temp <= (others => '0');

elsif rising_edge(clk) then

temp <= std_logic_vector(unsigned(temp) + 1);

end if;

end process;

sclk <= temp(22);

-- Counter

counter : process(sclk, reset)

variable m : integer range 0 to n-1 := 0;

begin

if (reset = '1') then

m := 0;

elsif rising_edge(sclk) then

if (m = n-1) then

m := 0;

else

m := m + 1;

end if;

end if;

dout <= std_logic_vector(to_unsigned(m,7));

end process;

end A_modN;

Constraint file UCF code.

NET "clk" LOC = "L15" | IOSTANDARD = LVCMOS33;

NET "reset" LOC = "E4" | IOSTANDARD = LVCMOS33;

NET "dout<0>" LOC = "U18" | IOSTANDARD = LVCMOS33;

NET "dout<1>" LOC = "M14" | IOSTANDARD = LVCMOS33;

NET "dout<2>" LOC = "N14" | IOSTANDARD = LVCMOS33;

NET "dout<3>" LOC = "L14" | IOSTANDARD = LVCMOS33;

NET "dout<4>" LOC = "M13" | IOSTANDARD = LVCMOS33;

NET "dout<5>" LOC = "D4"  | IOSTANDARD = LVCMOS33;

NET "dout<6>" LOC = "P16" | IOSTANDARD = LVCMOS33;